- Technical Support – Work Order Specification

Title: Provision of Technical Support Assessing the Fatigue Crack Initiation Methodology

Background to the project The licensee, NNB Generation Company (HPC) Limited (NNB GenCo), further known as the HPC Licensee, submitted the closure form for Generic Design Assessment Finding (GDAF) SI-06 to the Office for Nuclear Regulation (ONR) to provide the evidence to support the resolution of a concern raised during Generic Design Assessment (GDA) and the basis for closure of GDAF SI-06. GDAF SI-06 states that “The Licensee shall engage with ND to ensure that the fracture assessment procedure used to calculate the limiting defect sizes will be suitable for supporting a UK based safety case” This GDAF was raised in the Structural Integrity GDA Step 4 report. It was noted that the requesting parties had based their fracture assessment methodologies on the French developed methodology (RSE-M), known as the UK methodology. This differs from the approach generally adopted in the UK to date where Licensees have undertaken fracture mechanics assessments (FMA) to the R6 Procedure. The HPC Licensee commissioned an independent review of the RSE-M fracture mechanics code whose aim was to provide greater assurance of the provenance of the code and its applicability to the UK regulatory environment. In the majority this review provided confidence in the proposed UK methodology. However, the UK methodology proposes to use the RSE-M guidance for Fatigue Crack Initiation (FCI) and Fatigue Crack Growth (FCG) as given in Appendix 5.3 & 5.6 of RSE-M. The use of FCI is a departure from the UK experience of FMA approach. Currently the ONR is not familiar with the use of a FCI as there is a limited understanding on the implication and levels of conservatism.

SCOPE OF THE SERVICES REQUIRED As described above the ONR is unfamiliar with the FCI approach proposed and wishes to develop a greater understanding of the mechanics of method; the application in a UK Safety Case and the uncertainties that are present within the supporting validation. The work is for the Technical Support Contractor (TSC) to advise ONR if, in their professional opinion, the basis of the FCI methodology as outlined within the UK methodology is adequate to support a High Integrity Component (HIC) claim. - Task 1: Understand the FCI approach as detailed within the UK methodology The aim of this phase is to read and understand the basis of the FCI approach as detailed within the UK methodology and the RSE-M code. This should include both the mechanisms within RSE-M Appendix 5.3 and material data laws detailed in RSE-M Appendix 5.6. It is expected that this will include informal discussions with ONR inspectors to aid a full understanding. - Task 2: Literature review of the UK & International use of the FCI methodology or similar approaches The aim of this phase is to conduct a literature review on the use of the FCI methodology or similar approach within the UK and International community (e.g. the R5 sigmad or TASGI review of non-sharp defects). This should primarily focus on nuclear applications but, if appropriate, information from other industries may be included. It is expected that precedents for the use of the FCI or similar approaches would be identified and opinions be documented. - Task 3: Evaluate the validity of the proposed FCI methodology as detailed within the UK methodology. The aim of this task is to digest and understand the supporting evidence that underpins the FCI methodology. It is expected that the experimental data will be reviewed and uncertainties in the basis of this experimental data and any extrapolation from the test data be highlighted. - Task 4: Preform illustrative calculations using both the UK Methodology and R6 approaches to crack extension through fatigue and identify the differences. The aim of this task is to provide an illustrative comparison of the approaches proposed in the UK methodology and R6 to estimate the amount of crack extension induced by fatigue loading. It has been assumed that any differences can be highlighted using three different examples, which will consider the impact of different configurations (e.g. component geometries and loading variations). It is expected that the TSC will proposed the different cases to be examined and to provide a rationale behind why these cases have been chosen. If the TSC considered that additional cases are required then the justification should be presented. To complete these assessments the TSC should identify if they have suitable transient input information or if they require this to be provided. - Task 5: Create report It is expected that the TSC will write a report detailing the activities of the previous three phases, including appropriate drawings and figures to support the discussion. This report should clearly state the uncertainties inherent in the FCI approach proposed. In addition, this report should clearly state the professional opinion of the TSC on whether this approach meets the expectations laid out for a High Integrity Component and whose failure could be discounted from the design basis. - Task 6: Presentation of Results Once the report has been written it is expected that the TSC will make themselves available to discuss the details of the report and answer any clarification questions..

OBJECTIVES The overall objective of this work is for the TSC to aid ONR in making its regulatory decision on the suitability of FCI methodology in supporting a HIC claim by reviewing the documents and capturing their advice, and reasons for this advice, in a report.

CONSTRAINTS The volume of the supporting information is unknown and therefore ONR understands the difficulty in estimating the level of work required. ONR would suggest that the TSC looks at the topic and makes a judgement on the complexity of the issue and bases the estimate on this. Once the documents are transmitted, the timescales can be reviewed with the successful TSC. It is therefore recommended that the TSC makes it clear in the tender documentation what effort has been estimated in this phase. Due to the nature of the work, the personnel involved in this work cannot currently be working on any projects related to HPC. If the personnel identified have previously worked on any of the documents being assessed this should be clearly stated as a potential conflict of interest. Consideration should also be given to the working environment etc if others in the office where the work is to take place, are currently working on HPC projects. It is expected that the process from receiving the information to production of the report should take no longer than 3 months; therefore, the tender should show how the TSCs intend to manage the work in order to meet this timescale. The staff proposed for this work must be able to demonstrate that they have a Fatigue testing and analysis background and have relevant experience in this topic. The security markings of the supporting information is not yet known, the process for transmission to the successful contractor will be determined once the ONR receives them.

CONTRACT MANAGEMENT ONR will require to be kept updated about progress and delivery of the required work via monthly meetings, to include a contract start-up meeting at ONR’s offices at Bootle or contractor’s premises where appropriate. Subsequent progress meeting should be arranged with the ONR Project Office and can be held at ONR’s office in Bootle, or the contractor’s premises.

TECHNICAL RESPONSE

Response - The tender should include the following:- - a description of how the work will be delivered and the proposed resources; - the CVs and details of all staff who will be undertaking the work; - the arrangements for managing delivery of the work and dealing with disputes (including details of who will act as the liaison point between the contractor and ONR); - a project plan showing the tasks under each phase and including meetings with ONR; - a description of the TSC’s understanding of each of the phases outlined in section 2. - details of how the charges for undertaking the required work has been calculated in a manner that enables ONR to understand and compare the costs, regardless of how the service is delivered. This should clearly show the assumptions used, especially related to the complexity of each AF.