Specifications for new minibus:

Model: Boxer L4H2 440 2.0 BlueHDi 130bhp 6 speed, Euro VI

Colour: Please specify colour/s available

Variant: CanDrive Easyon 4.25ton GVW Type: 17 seat Minibus, Up to 5 wheelchairs

Base Vehicle Specification:

 4 way heated electric mirrors & Electric windows

 Remote central locking with keyfob

 Blue tooth connectivity, DAB FM Radio with USB input

 Over head storage shelf

 Antilock Braking System, Electronic Brake Force Distribution and Electronic Stability Package

 Thatcham Immobiliser

 Lumbar and height adjustment on driver's seat, driver's seat armrest, height adjustable steering wheel

 Glazed heated rear doors

 Driver + double front passenger seat

 AEBS (Advanced Emergency Braking System)

 Lane departure warning system

 Digital Tachograph

 Peugeot 3 year contractual warranty with 1 year Peugeot Assist cover

Conversion:

 Fully glazed all round with kite marked safety glass, inc 2 top sliding windows as standard

 Rear saloon fully soft trimmed in transport grade Hi Flex velour carpet

 Emergency exit roof hatch

 Roof trimmed and insulated

 Rear saloon floor finished in transport grade non slip vinyl matting

 14 x M2 tested/certified rear saloon seats, fitted with grab handle style PU headrest and 3 point all age adjustable seat belt, vinyl seat trim/cushion/incontinence base

 All 14 rear seats mounted on quick release fittings so are removable to carry up to 5 wheelchair occupants

 Fully tracked flooring system

 PLS Access Lite externally mounted under floor hydro / electric wheelchair lift

 Twin full height Hi Visibility handrail & integral twin fold out side step inside sliding door entrance

 Low energy LED saloon lighting, step entrance lighting and LED strip light at step edge

 All legal lettering/signage

 Emergency exit break glass hammers

 1st aid kit & 2 x fire extinguishers

 Peugeot Approved GVW up rate to 4250kgs

 1 x set of Unwin heavy duty 4 point wheelchair restraints

 Restraint storage behind each inner wheel arch

 Full VCA / VOSA Type Approval and IVA tested/certified, 1st registered as Minibus

 3 year contractual warranty (some components may be excluded as covered by separate manufacturer guarantee). Please give details of warranty.

 Delivery (Rode Heath, Cheshire) and vehicle handover / familiarisation

Extras required:

Tracking (as floor tracking) on inside walls of vehicle (to fasten items using Unwin restraining belts)

Touch screen sat nav,

c/d player,

Front air con

Power folding mirrors

Rear heater

Rear air con

CCTV front, rear & internal

DVLA 1st Reg

Livery (“ Rode Runner Community Transport”  “0800 020 9614” ORPC logo and “supported by Cheshire East Council “ logo  on both sides of bus. ORPC will provide both logos as PDF or jpeg).