Specification – Canning Line:  Reference BUS001

## Background

Since planting our first trees in 1986 we've grown to become the largest independent cyder-maker in Cornwall and the first distillery for 300 years. Family owned and Cornish born and bred.

We are wishing to expand our offering by creating an in-house canning line.

We aim to procure a new, or ex demonstration, modern 6000 can per hour canning line.  This will consist of a ‘de-palletiser’ which unloads the empty cans from a pallet into the machine, and a ‘uniblock’ which will rinse, fill and seam the can.  Aiming to keep our capital costs to a minimum, we then plan to utilise the existing tunnel pasteuriser, labeller and end of line packing machines that already exist for glass bottles.

## Minimum Specification

- Installation to be completed by 31 December 2024.

- Work to be carried out during normal working hours.

- Compatible with our existing systems

- 6,000 CPH x 500ml

- 6,000 CPH x 250ml

- De-palletiser:

  - 12,000 units per hour

  - Fully automated sweep system for cans

  - Accommodate all pallet sizes

  - Drives & Motors

  - Pneumatic controls

  - Full CE certification

  - User friendly touchscreen control

  - Can accommodate all can sizes with simple sensor adjustment

  - Complete safety and interlock system

  - Twist and rinse module to suit other cans

  - Automated interlayer removal

  - Interlayer catchment module with bin

- Counter Pressure Filler/ Seamer

| - Type of filling | - Counterpressure with electro-pneumatic valves |
| --- | --- |
| - Production (cph) | - Up to 6000 cph @250ml - Up to 6000 cph @500ml |
| - Precision | - +/- 2 mm |
| - Safety covers | - According to CE rules |
| - Standard version | - Inlet can detector; - Overloading detectors; - Outlet conveyor (of 2 m length); - Regulator of the conveyor’s working speed synchronized with the machine speed; - PLC – driver B&R |
| - Noise (dBa) | - 86 |
| - Air pressure (Mpa) | - 0,8 |
| - Air consumption (m3/h) | - 8 |
| - Pressure of CO2 (Mpa) | - 0,8 |
| - Consumption of CO2 (kg/h) | - 20-50 |
| - Power supply (kW) | - 3,5 |
| - The volume required for sanitary cleaning (l) | - 220 |
| - Washing pressure (Bar) | - 3 |
| - Max. Temperature of washing (°C) | - 65 |
| - Legs (pcs) | - 10 of 120 mm diameter |

- Seamer (closure) 2 Heads

  - can pitch: 123 or 127 mm.

  - range speed: from 20 to 100 cpm (continuously adjustable)

- Software

- Filler Block

- X-ray fill level checker plus safety protection

- Ejection verification

- Vacuum pressure control unit (after tunnel pasteurisation)