Ground Floor Alterations - Business Support

The proposed alterations to the existing room layouts within business support, are sympathetic to the existing steel frame structure, incorporating the frame where possible.
No modifications are to be made to the steel frame, and any columns that do not fall within a new wall line are to be protected & boxed in.
The existing walls/infill panels are constructed out of 140mm concrete blocks, built tight to the underside of the steel beams/floors, but is considered as infill and not assisting the frame.

The removal of any walls which butt up tight to the steelwork, will result in leaving exposed steel, this will need either intumescent paint being applied or boxing in.
New fire exit (G19 Office) constructed in cavity wall brick/block construction, and built tight up to underside of existing first floor structure, to maintain a secure perimeter.
New cavity lintel required in forming new fire door to external wall of the old reception lobby/ new academic support space.
Layout to new gallery space/registry exams (old post room) to be adjusted to take into account of existing column missed of proposed floor layout.
Further existing columns within Admissions, Registry 7 & Language Dept, also need to be added back to proposed floor layout.