| client | Milton Keynes Development Partnership |
| --- | --- |
| Address | Milton Keynes Station Square, MK9 1FZ |
| contract title | Milton Keynes Station Square |
| operating unit |  |
| Project Number | 30229848 |
| Project Location | Milton Keynes Station Square |

Document Title

MILTON KEYNES STATION SQUARE PRE-CONSTRUCTION HEALTH & SAFETY INFORMATION

|  | NAME | CONTACT DETAILS |
| --- | --- | --- |
| Author | Tim Goddard | tim.Goddard@arcadis.com |
| Reviewer | Andy Wilnycki | Andy.wilnycki@arcadis.com |
| Reviewer | Sucha Panesar | Richad.Mothersdale@arcadis.com |
| Approver | Kevin Tapp | Kevin.tapp@arcadis.com |

| DOCUMENT No. | 30229848-ARC-GEN-##### |
| --- | --- |

| CURRENT REVISION | CURRENT STATUS CODE |  | SECURITY CLASSIFICATION |  |
| --- | --- | --- | --- | --- |
| P01 |  | First Issue |  |  |  |

| REVISION HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| REVISION |  | STATUS | DATE | REVISION DESCRIPTION |
| P01 |  |  | 18.10.2024 | First Issue |

ISSUE CONTROL

- CONTROLLED COPY DISTRIBUTION LIST

| Controlled Copy No. | Issued To: | Job Title: | Organisation: |
| --- | --- | --- | --- |
| 01 |  |  |  |
| 02 |  |  |  |
| 03 |  |  |  |
| 04 |  |  |  |
| 05 |  |  |  |
| 06 |  |  |  |
| 07 |  |  |  |
| 08 |  |  |  |
| 09 |  |  |  |
| 10 |  |  |  |

- Contents

1	Introduction	4

2	Client Health and Safety Standards and Requirements	5

2.1	Appointments	5

2.2	Design Phase	5

2.3	Mobilisation for Construction	6

2.4	Construction Phase	7

3	Project Information	8

4	Project Programme	11

5	Appointments and Notifications	12

6	Existing Health and Safety File, Records and Plans	14

7	Site Constraints and Operational Hazard Information	18

8	Significant Health and Safety Risks Identified During the Design Phase	20

9	Health and Safety File	23

10	Additional Information	24

10.1	Nearest A&E Hospitals	24

10.2	Emergency Evacuation Plan	24

10.3	Adjacent Projects	25

10.4	Environmental Considerations	28

10.5	Traffic & Pedestrian Management	28

10.6	Security	29

10.7	Access & Egress	29

- Appendices

Appendix A – Designers Risk Register

# Introduction

The Pre-Construction Information (PCI) provides the health & safety (H&S) focus for the design and construction phases of a project.  It sets out to ensure that the design and construction of the works are undertaken in a safe way that complies with H&S legislation, including the Construction (Design and Management) Regulations 2015.

This document constitutes the Pre-Construction Information and reflects the available information at the date of production and the concurrent stage of the design development.  It should be noted, however, that the design will continue to develop , and therefore the Pre-Construction Information will therefore continue to evolve.

# Client Health and Safety Standards and Requirements

The ‘Client’ under the Construction (Design and Management) Regulations 2015 (CDM regulations) has considerable influence over the health and safety performance achieved on construction projects which we undertake in their name.

Arcadis, working with the Client, is committed to ensuring that the underlying principles and requirements of the CDM Regulations are fully implemented on all construction projects so that exemplar standards of health and safety performance are achieved across our projects.

In order to assist the Client to fulfil their duties and to demonstrate our compliance with the requirements of the CDM Regulations, we have developed processes, procedures and checklists to be used at all work stages throughout a construction project.

## Appointments

The Principal Designer, Designers and Principal Contractor appointed by the Client have been, or will in time be, approved and appointed via the Clients Competency Review Process.

## Design Phase

This process requires nominated staff and where necessary external approved design professionals to review and approve designs, ensuring that they satisfy the Health and Safety Executive (HSE) ‘Principles of Prevention’.

It is expected that all Designers be familiar with the HSE Principles of Prevention and demonstrate that they have been adopted in all design work undertaken on behalf of the Client.

Health and safety must be an agenda item at all meetings, and it is essential that reviews take place during the design phase to identify strategies for safe access for construction and consideration for cleaning and maintenance of the projects. The Client will remain to be consulted and included in the development and agreement of these strategies.

During the design phase health and safety hazards associated with:

- Statutory Undertakers Equipment (Public & Private);

- Traffic management / Segregation of the Public / Pedestrian movements;

- Congested Pedestrian and Vehicular area;

- Use and access to adjacent buildings (inc. deliveries);

- Plant movements;

- Pedestrian movements;

- Maintenance;

- Refurbishment / Repair / Alteration / Change alignments.

- Must be identified and measures to reduce and control the level of risk, so far as is reasonably practicable.

A Designers Risk Management Schedule (DRMS) is available for review and assessment by the Principal Designer throughout the project to record significant and unusual hazards associated with the project, actions taken to eliminate, reduce or control the level of risk during design and residual risks that must be controlled during the construction and on-going use.

The DRMS is included within the Pre-Construction Information provided to the Principal Contractor / Contractors (Appendix A).

Principal Contractors, note that significant and unusual hazards which

have not been eliminated during the design process are to be annotated

using pictorial warnings supported with written dialogue in the Safety,

Health and Environmental Information Box on design and construction

drawings provided to the Principal Contractor / Contractors.  These

hazards have also been included in the DMRS.

This allows for all relevant risk information to be collated in one place and ensures all significant and unusual issues are not missed during design changes.  This practise also ensures that the intended audience, the contractors and their employees will visually be made aware of these issues.

## Mobilisation for Construction

No work is to commence until the Principal Contractors Construction Phase Plan has been assessed for its adequacy by the Client.

The Principal Contractor Construction Phase Plan must be sufficient to address clearly, the arrangements for managing and organising the project and include information as listed in the ‘Construction Phase Plan Essential Information’, see HSE L153 prior to work commencing on site.

The Construction Phase Plan must make reference to the health and safety competence reviews of any and all sub-contractors used for the project. The Plan must contain an example of the process used to undertake this evaluation, i.e. questionnaire / evaluation form contractors are required to complete.

The content of any existing Health and Safety File and the survey reports and utilities information provided with this Pre-Construction Information must be referenced and addressed within the Principal Contractor / Contractor’s Construction Phase Plan and project specific Risk Assessments and Method Statements.

## Construction Phase

On receipt of authorisation to proceed the Principal Contractor is under a legal duty to administer, implement and update the Construction Phase Plan (CPP) as required by changes in design or circumstances, throughout the construction period.

The Principal Contractor and all Contractors must ensure that welfare facilities are in place and all operatives are familiar with the Site Induction and Construction Phase Plan before starting work on site and comply with the requirements at all times.

A copy of the DRMS is provided in Appendix A.

Health and safety must be an agenda item at all site progress meetings, to ensure that the necessary liaison takes place during the construction phase to identify, amend and implement risk control measures to ensure the continuing health, safety and welfare of site staff, customers and other visitors to the premises.

To ensure that Principal Contractors and Contractors continue to manage health and safety to a high standard on site throughout the construction phase of projects, the Principal Contractor will arrange for site health and safety inspections to be carried out.

# Project Information

During this project the single point of contact for any Client queries or discussions is:

Client

Lisa Noall Civic Offices, 1 Saxon Gate East, Milton Keynes MK9 3EJ

Principal Designer

Tim Goddard Arcadis Consulting (UK) Ltd 80 Fenchurch Street London EC3M 4BY

Designer

Andy Walnycki Arcadis Consulting (UK) Ltd 80 Fenchurch Street London EC3M 4BY

Designer - Landscape

Hannah Vincent 2 Back Grafton Street, Altrincham, Cheshire, WA14 1DY

Designer – Irrigation

Paul Dawn-Bishop Waterscapes Limited 1 Murray Court Wincanton Business Park Wincanton Somerset England BA9 9RX

Designer – Artwork

To be confirmed and notified to the successful contractor

Principal Contractor

TBC

Location where work will take place:

Scheme Name: Milton Keynes Station Square Area: Milton Keynes Highway Authority: Milton Keynes City Council OS Map Reference: SP 84294 38109 Nearest Postcode: MK9 1LR Eastings: 484294 Northings: 238109

Provide a brief description of the works to be carried:

Improvements to Station Square in central Milton Keynes replacing existing paving, new landscaping and public artwork to enhance the gateway between the railway station and the city centre. Further details are available via the following: 30229848-ARC-HGN-ZZ-DR-HE-00001 – General Arrangement Milton Keynes Station Square 30229848-ARC-HML-ZZ-DR-HE-00002 - Long Sections Sheet 1 of 3 Milton Keynes Station Square 30229848-ARC-HML-ZZ-DR-HE-00003 - Long Sections Sheet 2 of 3 Milton Keynes Station Square 30229848-ARC-HML-ZZ-DR-HE-00004 - Long Sections Sheet 3 of 3 Milton Keynes Station Square 30229848-ARC-HKF-ZZ-DR-HE-00001- Proposed Kerbs Footways and Paved Areas Milton Keynes Station Square 30229848-ARC-HML-ZZ-DR-HE-00001 - Setting Out Details Milton Keynes Station Square 30229848-ARC-HSC-ZZ-DR-HE-00001 – Site Clearance Milton Keynes Station Square 30229848-ARC-GEN-ZZ-DR-HE-00001 – Standard Details Milton Keynes Station Square

# Project Programme

| Date for commencement of design period: | 10th June 2024 |
| --- | --- |
| Time period allocated for design (weeks): | 19 weeks |
| Date for appointment of Contractor / Principal Contractor: | 20th December 2024 |
| Time period allocated for mobilisation by Contractor / Principal Contractor (weeks): | 4 weeks |
| Date for commencement of construction period: | 3rd February 2025 |
| Time period allocated for construction (weeks): | 20 weeks |

The final agreed programme, giving the timescales for the commencement, phasing, sectional completion, final practical completion and commissioning of the works, is to be incorporated within the Construction Phase Plan.

# Appointments and Notifications

| Will the work require more than one contractor to be working on the project at any time? | YES |  |
| --- | --- | --- |

If “YES” insert Principal Designer and Principal Contractor details below.

If “NO” insert Contractor details below.

| PRINCIPAL DESIGNER |  |
| --- | --- |
| Company: | Arcadis Consulting (UK) Ltd. |
| Contact Name: | Tim Goddard – Principal Designer Manager |
| Contact Details: | Tim.goddard@arcadis.com |
| PRINCIPAL CONTRACTOR |  |
| Company: | TBC |
| Contact Name: | TBC |
| Contact Details: | TBC |

Details of other Key Project Participants (e.g. Project Manager, Designers, Contractors)

| Role | Company | Contact name | Contact Details |
| --- | --- | --- | --- |

| Do the works require notification to the HSE? (30 days and more than 20 people on site at any one time or 500 man days, e.g. 50 people for 10 days) | YES |  |
| --- | --- | --- |

| If “YES” insert date F10 submitted to the HSE: | TBC |
| --- | --- |

# Existing Health and Safety File, Records and Plans

| Is there an existing Health and Safety File available? (please delete as applicable) |  | NO |  |
| --- | --- | --- | --- |
| If ‘YES’ above, the Health and Safety File is available at: | Milton Keynes Development Partnership have issued available information in relation to these works which has been reviewed in the current design |  |  |

| Is there a site layout plan available? (to assist the Contractor in planning their access routes, location of equipment and materials storage areas and emergency arrangements - please delete as applicable) |  | YES |  |
| --- | --- | --- | --- |
| If ‘YES’ above, a site layout plan is available at: | 30229848-ARC-HGN-ZZ-DR-HE-0001 – General Arrangement Milton Keynes Station Square. |  |  |

| Asbestos Survey It is essential that the area of work be demonstrated to be free of asbestos or a Refurbishment / Demolition type survey for asbestos has been undertaken that covers the part/s of the works. |  |  |  |
| --- | --- | --- | --- |
| Can the area of work be demonstrated to be free of asbestos? Evidence available: |  |  | NO |
| If “NO”, has a Refurbishment / Demolition type survey been carried out that covers the part/s where the project will take place? |  |  | NO |
| Surveying Company: | N/A |  |  |
| Surveyor Name: |  |  |  |
| Survey Report No: |  |  |  |
| Date of Survey: |  |  |  |
| Survey Report is available at: |  |  |  |

| Surveys and Health and Safety Related Documents The following additional surveys and reference documents are available for this project: |  |
| --- | --- |
| Report Title & Reference: | Comments: |
| 30229848-ARC-HGN-ZZ-DR-HE-0001 | General Arrangement – Milton Keynes Station Square |
| 30229848-ARC-HGN-ZZ-DR-HE-0002 | Constraints Plan – Milton Keynes Station Square |
| 30229848-ARCHSC-ZZ-DR-HE-00001 | Site Clearance – Milton Keynes Station Square |
| MKC-RINWAY-STATION SQUARE-PAS128-TYPE-B-250124 | GRP PAS128 Survey Results |
| 240125-MKC-RW-STN | Topographical PAS128 Survey – Milton Keynes Station Square |
| Station Square Trial Hole Data | Trial Hole Log Sheets 1 – 6 for Milton Keynes Station Square |
| 2023-24/Station Square Trial Holes/01ª | Trial Hole Locations Plan – Milton Keynes Station Square |
| 30229848-ARC-HGN-01-TH-ZZ-00002 | Design Risks Management Schedule – Milton Kynes Station Square |

| Information available, e.g location drawings, report, etc | Available at: |
| --- | --- |
| MKC-RINWAY-STATION SQUARE-PAS128-TYPE-B-250124 | GRP PAS128 Survey Results |
| 30229848-ARC-HGN-ZZ-DR-HE-0002 | Constraints Plan – Milton Keynes Station Square |
| 1508804-1 (02/07/2024) | Anglian Water – Clean Water C2 Drawing |
| 1508804-2 (02/07/2024) | Anglian Water - Wastewater C2 Drawing |
| QPB09143F (02/07/2024) | BT Openreach – C2 Drawing |
| CFH_EP_000001 (02/07/2024) | CityFibre Existing Plant – C2 Drawing |
| LSBUD Ref: 33922360 (02/07/2024) | Linesearch Enquiry – C2 |
| 33922360 (02/07/2024) | National Grid Electricity Ditribution - C2 Drawings (7no.) |
| 33922360 (02/07/2024) | Southern Gas Network - C2 Drawings (3no.) |
| LSBUD Ref: 33922360 (02/07/2024) | Squire Energy Networks - C2 |

Provided within this Pre-Construction Information (Table above) are the Utilites information gathered during the design stage.  Prior to work commencing the Principal Contractor shall undertake further investigations, as required, to ensure they have in their possession the most up to date information and not be reliant on the design phase utilities information.

The Principal Contractor shall make arrangements with the Statutory Undertakers and others concerned, for the coordination of his work with all work, which needs to be done by them or their contractors concurrently with the Works.  Compliance with the expiration periods of the Statutory Undertaker provided documentation does not relieve the Contractor of his obligations as a Principal Contractor.  It is the responsibility of the Principal Contractor to ensure they have sufficient information to undertake excavation safely prior to the start of works and Ground Penetrating Radar Systems (GPRS) / Groundbreaking safe system of works should be carried out prior to any excavation work.

# Site Constraints and Operational Hazard Information

| The permitted core working hours for construction work will be: |  |
| --- | --- |
| Monday – Friday: | 07:00 – 18:00 |
| Saturday: | 08:00 – 13:00 |
| Sunday: | Not permitted without authorisation |
| Bank Holidays: | Not permitted without authorisation |

Additional hours may be permitted subject to prior agreement by the Project and conforming to the local authorities noisy work restrictions.

| Welfare Arrangements |  |
| --- | --- |
| Toilets: | TBC |
| Hand Washing: | TBC |
| Rest area: | TBC |
| Canteen: | TBC |

| Materials Delivery, Storage Arrangements and Parking |  |
| --- | --- |
| Where must project materials be delivered to? | TBC |
| Are there any restrictions on delivery times for materials? |  |
| Where must project materials be stored? Please identify if there is no storage area available. | TBC |
| What parking is available at the site for Contractors vehicles? Please identify if there is no Contractor parking area available or the location of the designated parking. | Public car parks and within the contractors site compound |

# Significant Health and Safety Risks Identified During the Design Phase

| Site Specific Health and Safety Hazards The following site-specific constraints and hazards have been identified as significant within the design risk register. The full list of health and safety risks is contained within the Design Risk Management Schedule in Appendix A. 30229848-ARC-HGN-01-TH-ZZ-00002 – Design Risk Management Schedule Milton Keynes Station Square |  |
| --- | --- |
| Hazard/Risk | Details |
| 'HS/S1/002 Excavation and Construction footway in or near underground utilities including known Gas and Electrical plant Underground Utilities | 'To reduce the risk a 3D GPR survey has been provided undertaken 'MKC-RINWAY-STATION-SQUARE-PAS128-TYPE-B-250124' and C2 requests for all affected plant have also been provided '32224_04_00 Utility Searches' . During design we have kept to existing levels do cover to existing utilities will be no worse than it is currently. The risk during construction also cannot be eliminated as excavation need s to take place near or adjacent to utilities and the GPR survey and C2 plans may not be wholly accurate. The contract shall ensure all excavations are scanned with a cable avoidance tool and that all utilities have been identified and confirmed with utility providers prior to any excavation. The contractor shall also consider hand dug trails holes and vacuum excavation to reduce the risk of hitting existing utilities. 'There remains a risk of hitting existing utilities as they may not be at the location shown on the GPR survey 3D GPR survey has been provided undertaken 'MKC-RINWAY-STATION-SQUARE-PAS128-TYPE-B-250124' or C2 plans provided ( '32224_04_00 Utility Searches' ) 30229848-ARC-HGN-ZZ-DR-HE-0002 – Constraints Plan – Milton Keynes Station Square |
| 'HS/S1/005 Site compound delivery of materials and access to site for large and medium goods vehicles during construction. | Site access and egress is required which enters and exits from a busy ring road around the site with access to be maintained to the rail station and bus station hence the risk to delivery vehicles and access to the site compound cannot be wholly eliminated . The risk can be reduced by ensuring that suitable and sufficient access and egress points are set up within the site and areas of loading this may require temporary access road and phasing of the works to ensure the installed pavement is not damaged and that loading vehicles do not create queuing back onto public roads. Traffic management should be installed maintained and removed in accordance with The red book safely at street works and roadworks and Chapter 8 traffic management. |
| HS/S1/006 Basement surveys underground structures Collapsing ground | The risk of underground basements was discussed with the client and they stated that the previous land use was farmland and a GPR survey 'MKC-RINWAY-STATION-SQUARE-PAS128-TYPE-B-250124' has been completed that did not detect any underground voids or basements.The risk is therefore reduced but there is still a risk of voids that have not bene detected by the GPR survey. |
| HS/S1/008 Asbestos ducting and drainage | The client has stated that there are no records available to determine the presence of asbestos on site. However, the site was constructed in the 1980s and the utility records do not show any asbestos cement pipework so the risk is reduced to an extent. IT is also know that the previous site usage was farmland. However there remains a risk of asbestos from demolition waste used in the subbase or subgrade of the existing footway . Trial holes undertaken by Eurovia in January 2024 have been undertaken within the site and found clay and type 1 sub-base hence the overall risk is reduced . There remains a risk that asbestos may be uncovered during excavations and the contractor should ensure they follow the Asbestos management regulations if any material is uncovered. Principal Contractor to ensure competence resource available to witness excavations to identify potential Asbestos / Contamination. |

# Health and Safety File

The health and safety file is defined as a file appropriate to the characteristics of the project, containing relevant health and safety information to be taken into account during any subsequent project.

The file must contain information about the current project likely to be needed to ensure health and safety during any subsequent work, such as maintenance, cleaning, refurbishment or demolition. When preparing the health and safety file, information on the following should be considered for inclusion:

- A brief description of the work carried out.

- any hazards that have not been eliminated through the design and construction processes, and how they have been addressed (e.g. surveys or other information concerning asbestos or contaminated land, utility interfaces);

- hazardous materials used.

- information regarding the removal or dismantling of installed plant and equipment (e.g. any special arrangements for lifting such equipment).

- health and safety information about equipment provided for cleaning or maintaining the project, limitations, maintenance strategy.

- the nature, location and markings of significant services, including underground utilities.

- information and as-built drawings of the project.

There should be enough detail to allow the likely risks to be identified and addressed by those carrying out the work. However, the level of detail should be proportionate to the risks. The file should not include things that will be of no help when planning future construction work such as pre-construction information, the construction phase plan, contractual documents, safety method statements etc. Information must be in a convenient form, clear, concise and easily understandable.

Where Asbestos Containing Materials have been identified and where the project will require work on, or which will disturb or is liable to disturb, materials containing asbestos the Construction Phase Plan must include specific measures for managing the risk and avoiding risk of exposure.

The Principal Contractor is to review all information required and procure all outstanding information, all in accordance with the Health and Safety File requirements.

The Principal Contractor is to notify the Principal Designer when they consider the Health and Safety File is ready for the Principal Designer to review.

# Additional Information

## Nearest A&E Hospitals

The nearest A&E Hospital to Milton Keynes Station Square is located 2.6 miles away and it is:

Milton Keynes Hospital

Standing Way

Milton Keynes

Buckinghamshire

MK6 5LD

Tel: 01908 660033

The map below (from Google Maps) shows the location of Milton Keynes Hospital in relation to the location of the site:

## Emergency Evacuation Plan

Due to the site location adjacent to Milton Keynes Railway Station it would be good practice to complete an emergency evacuation plan by the Principal Contractor.  This aim of the Emergency Evacuation Plan document is to describe the emergency procedure to be instigated during the works to address the likely impacts as follows:

•Unable to access working areas

•Adjustment situations within the immediate surrounding area / adjacent facilties

•Emergency backfill and reinstatement to allow use for the public and/or emergency services

•Additional pedestrian facilities.

Up to date contact information of all key personnel and relevant stakeholders is provided in the Emergency Evacuation Plan / site fencing.

## Adjacent Projects

Th Principal Contractor are to ensure liaison and co-ordination is undertaken with adjacent projects where either party, additional stakeholders or members of the public are affected.

The Principal Contractor are to continue to monitor the Milton Keynes City Council (MKCC) Planning portal during the construction period of the works to monitor potential adjacent projects.  MKCC Planning portal available via the following link.

View and comment on planning applications | Milton Keynes City Council (milton-keynes.gov.uk)

The Principal Contractor must ensure, in addition to co-ordination with adjacent projects and the client, they must ensure liaison of the works are regularly reviewed with the adjacent Milton Keynes Railway Station (Contact Centre Team – 0333 311 0006)

## Environmental Considerations

Contaminated ground - Should excavated material be suspected of being contaminated it must be tested and classified, Principal Contractor should take sufficient measures to isolate the contaminated (or suspected contaminated) material.  These measures shall include, but not necessarily be limited to, securely fencing off and clearly identifying to all personnel the location of any stockpiles or in-situ material, prior to identification of the type and concentration of any contaminants.

Noise – noise and vibration will be minimised by controlling it at source and/or fitting silencers and dampers where possible. Machinery will not be kept running unnecessarily.

All pneumatic tools are to be noise suppressed. Plant will be immobilised when not in use. General requirements associated with noise and vibration are identified the Principal Contractors Construction Phase H&S Plan.

Pollution – precautions will be taken to protect against pollution of watercourses and the air. The ground will be damped down to ensure that dust is not generated. During any cutting, dust will be kept to a minimum by damping down.

## Traffic & Pedestrian Management

Strict traffic management measures shall be implemented to ensure that the inconvenience to the public arising from increased traffic flows and the disruptive effects of construction traffic on surrounding roads is minimised and that continuing site activities have a minimal effect on the surrounding environment. The works will have a specific site logistics plan routing vehicles. The site logistics plans should be signed off by one of Principal Contractors management team who will hold adequate training for the approval of TMP’s prior to submission to the client.

The principal traffic management activities include:

- Movement of vehicles, equipment and materials on the highway in the immediate vicinity of the site, including wheel-washing and sheeting of vehicles as applicable.

- Transfer of spoil and waste.

- Management of traffic / pedestrians associated with the use of the adjacent Milton Keynes Railway Station.

The plan should include agreed delivery times, unloading options, public diversions, routes avoiding peak times, holding areas, a booking system, traffic marshals, banksman, site speed limit should be in place. It will be communicated to subcontractors, suppliers, visitors prior to their arrival on site.

Pedestrian-only areas and safe designated pedestrian routes to work locations from which vehicles are completely excluded must be established and maintained.

A review of long-term static Traffic management is to be undertaken by the Principal Contractor.

## Security

The objective of the security will be to ensure business continuity and to minimise damage and theft by preventing and/or minimising the impact of security incidents. The site perimeter should be secured from the onset of works.  The Principal Contractor will be solely responsible for the security into the site once Principal Contractor takes possession of the site.  In addition, entry to site should be strictly controlled.

The Principal Contractor will complete a Logistics plan for the works.

## Access & Egress

Works access and egress points to be utilised and restricted with only authorised personnel who are trained and competent allowed in the works area. Any visitors will be inducted and accompanied by the respective Principal Contractor personnel.

Appendix A – Designers Risk Register

30229848-ARC-HGN-01-TH-ZZ-00002 – MKSS – Design Risk Management Schedule