Specification for Little Waltham Pavilion

- Decorating:

Exterior – Front elevation

11 load bearing timber posts with cross rails and bench seating – to prepare and paint, with three coats

2 sets of french doors, a single door and garage door – to prepare and apply an undercoat and top coats

New fascia boards – to prepare, prime new timber/ply, followed by an undercoat and topcoat

Interior

- Kitchen

Ceiling – prepare, stain block, two coats of white emulsion

Walls – prepare, fill holes and cracks, prime where necessary and apply two coats of coloured emulsion

Woodwork – prepare door frames and skirting, apply undercoat followed by top coat in eggshell finish

- Utility area

Ceiling – to prepare, stain block, two coats of white emulsion

Walls – prepare, fill holes and cracks, prime where necessary and apply two coats of coloured emulsion

Woodwork – prepare door frames and skirting, apply undercoat followed by top coat in eggshell finish

- Open social area

Ceiling – to prepare, stain block, two coats of white emulsion, including faux beams

Walls – prepare, fill holes and cracks, prime where necessary and apply two coats of coloured emulsion, including faux beams

Woodwork – prepare door frames and skirting, apply undercoat followed by top coat in eggshell finish

- Rear hallway

Woodwork – to prepare door frames and skirting, apply an undercoat followed by top coat in an eggshell finish

- WC 1

Ceiling – to prepare, stain block, two coats of white emulsion

Woodwork – prepare door frames and skirting, apply undercoat followed by top coat in eggshell finish

- WC 2

Ceiling – to prepare, stain block, two coats of white emulsion

Woodwork – prepare door frames and skirting, apply undercoat followed by top coat in eggshell finish

- General Building Works:

- Remove door and frame to Lobby

- Form new 900mm opening to Lobby and install lintel

- Making good as required

- Plumbing

- Plumb in a new kitchen sink

- Supply and install new compliant DOC M suite to existing male WC

- Supply and install 2 new low level WCs and 2 wall mounted basin, taps and waste outlets

- White-rock half height to basin wall in all cubicles

- Supply and install unvented water heater and safety kits throughout

- Install hot and cold water service pipework as required in all bathrooms

- Install soil pipe connection for WC, waste trap and pipework for basin

- Install thermostatic mixing valves to all hot water outlets

- Install SCV’s to comply with water regulations

- Electrical works

- All electrical works required for plumbing

- All electrical works required in the Toilets, Lounge, Kitchen, Bar, and Stores.

- Flooring

- New flooring supplied and fitted, through out the Stores, Bar, Lounge, Hall and Toilets, and to dispose of old flooring.