R10	RAINWATER PIPEWORK/GUTTERS

To be read with Preliminaries/General conditions.

TYPE(S) OF PIPEWORK/GUTTER

R10/260	PVC-U PIPEWORK FOR EXTERNAL USE:

-	Pipes, fittings and accessories: To BS 4576:Part 1, Kitemark certified.

Nominal size(s): 68mm.

Colour: Black

Accessories: Stop ends, outlet connections, offset bends, joints and brackets.

-	Method of fixing: as per manufacturers instructions.

R10/280	PVC-U GUTTERS:

-	Gutters and fittings: To BS 4576:Part 1, Kitemark certified.

Size(s): 112mm

Colour: Black

-	Method of fixing: as per manufacturers instructions.

INSTALLATION

R10/400	BEFORE COMMENCING WORK specified in this section, ensure that:

-	Below ground drainage is ready to receive rainwater or that the discharge can be dispersed by approved means to prevent damage or disfigurement of the building fabric.

-	Any specified painting of surfaces which will be concealed or inaccessible is completed.

R10/410	INSTALLATION GENERALLY:

-	Install pipework/gutters to ensure the complete discharge of rainwater from the building without leaking.

-	Obtain all components for each type of pipework/guttering from the same manufacturer unless specified otherwise.

-	Provide access fittings and rodding eyes as necessary in convenient locations to permit adequate cleaning and testing of pipework.

-	Avoid contact between dissimilar metals and other materials which would result in electrolytic corrosion.

-	Do not bend plastics or galvanized steel pipes.

-	Adequately protect pipework/gutters from damage and distortion during construction. Fit purpose made temporary caps to prevent ingress of debris. Fit all access covers, cleaning eyes and blanking plates as the work proceeds.

-	Where not specified otherwise use plated, sherardized, galvanized or nonferrous fastenings, suitable for the purpose and background, and compatible with the material being fixed.

R10/415	BUILDERS WORK: Comply with restrictions on the cutting of holes, chases, notches, etc. and methods of attachment to the building fabric specified in section P31.

R10/417	PREPARATION OF EXISTING TIMBER BASES: Inform CA of any defective boards/sheets and comply with instructions for replacement. Ensure that all boards/sheets are securely fixed. Punch in any protruding fastenings and plane or sand as necessary to achieve an even surface.

R10/420	FIXING GUTTERS:

-	Set out to a true line and even gradient to ensure no ponding or backfall. Position high points of gutters as close as practical to the roof and low points not more than 50 mm below the roof.

-	Position outlets to align with connections to below ground drainage, unless shown otherwise on drawings.

-	Provide for thermal and building movement when fixing and jointing, and ensure that clearances are not reduced as fixing proceeds.

-	Seal as specified to make watertight.

-	Ensure that roofing underlay is dressed into gutter.

R10/430	GUTTER BRACKETS: Fix securely at specified centres and at all joints in gutters, with additional brackets near angles and outlets.

R10/450	RAINWATER OUTLETS: Ensure that:

-	Outlets are securely fixed before connecting pipework.

-	Junctions between outlets and pipework can accommodate all movement in the structure and pipework.

R10/460	FIXING PIPEWORK:

-	Fix securely at specified centres plumb and/or true to line.

-	Make changes in direction of pipe runs only where shown on drawings unless otherwise approved.

-	Fix branches and low gradient sections with uniform and adequate falls to drain efficiently.

-	Fix externally socketed pipes/fittings with sockets facing upstream.

-	Provide additional supports as necessary to support junctions and changes in direction.

-	Fix every length of pipe at or close below the socket collar or coupling.

-	Provide a load bearing support for vertical pipes at not less than every storey level. Tighten fixings as the work proceeds so that every storey is self supporting and undue weight is not imposed on fixings at the base of the pipe.

-	Isolate from structure where passing through walls or floors and sleeve pipes as specified in Section P31.

-	Provide for thermal and building movement when fixing and jointing, and ensure that clearances are not reduced as fixing proceeds.

-	Fix expansion joint pipe sockets rigidly to the building and elsewhere use fixings that allow the pipe to slide.

R10/465	JOINTING PIPEWORK/GUTTERS:

-	Joint using materials, fittings and techniques which will make effective and durable connections.

-	Joint differing pipework/gutter systems with adaptors recommended by manufacturer(s).

-	Cut ends of pipes to be clean and square with burrs and swarf removed. Chamfer pipe ends before inserting into ring seal sockets.

-	Ensure that jointing or mating surfaces are clean, and where necessary lubricated, immediately before assembly.

-	Form junctions using fittings intended for the purpose ensuring that jointing material does not project into bore of pipes, fittings and appliances.

-	Remove surplus flux/solvent/cement/sealant from joints.

R10/530	IDENTIFICATION OF INTERNAL RAINWATER PIPEWORK: To BS 1710 using self-adhesive bands or identification clips located at junctions, at both sides of each slab, bulkhead and wall penetration, and elsewhere as directed.

R10/540	ACCESS FOR TESTING AND MAINTENANCE:

-	Install pipework and gutters with adequate clearance to permit testing, cleaning and maintenance.

-	Position access fittings and rodding eyes so that they are not obstructed by other pipework, framing, etc.

R10/550	TESTING GENERALLY:

-	Inform CA sufficiently in advance to give him a reasonable opportunity to observe tests.

-	Check that all sections of installation are free from obstruction and debris before testing.

-	Provide clean water, assistance and apparatus for testing as required.

-	Carry out tests as specified. After testing, locate and remedy all defects without delay and retest as instructed.

-	Keep a record of all tests and provide a copy of each to the CA.

R10/570	GUTTER TEST: Block all outlets, fill gutters to overflow level and after 5 minutes closely inspect for leakage.

R10/590	IMMEDIATELY BEFORE HANDOVER:

-	Remove construction rubbish and debris from all roofs and gutters. Where possible, sweep and remove fine dust which may enter rainwater systems. Do not sweep or flush dust or debris into the rainwater system.

-	Remove swarf, debris and temporary caps from the entire rainwater installation.

-	Ensure that all access covers, rodding eyes, outlet gratings, etc. are secured complete with all fixings.