# ANNEX E :TENDER SPECIFICATION – TOILET REFURBISHMENT

Project: 	Toilet Refurbishment, Bawtry Town Hall, Station Road, Bawtry, DN10 6PU

Client: 		Bawtry Town Council

Date: 		3rd November 2025

## 1. PROJECT OVERVIEW

The project comprises alterations to the existing ground-floor toilet areas to create new sanitary facilities including provision of:

- One separate disabled toilet (DDA/Equality Act compliant)

- Ladies WC room with three toilets, three sinks, and one hand dryer

- Gentlemen’s WC room with two toilets, two urinals, three sinks, and one hand dryer

- New flooring throughout (including foyer/hall entrance), plumbing, electrical, ventilation, partitions, signage, decoration, testing & commissioning.

2. REFERENCE DOCUMENTS

- Proposed and Existing layout plan (Annex E of the tender document)

- UK Building Regulations

- Relevant British Standards

## 3. SITE ESTABLISHMENT

## Contractor shall provide welfare, protection, access, safety, and storage provisions as required.

## 4. DEMOLITION & STRIP-OUT

## Remove partitions, internal doors within the toilets (not toilet entrance doors), fixtures, sanitary ware, finishes, and redundant services. Cap services safely as may be required and dispose of legally.

5. BUILDING/SCOPE OF WORK

- Construct new wall layout in accordance with proposed drawing

- Modify partitions to suit layout shown in drawing

- Install accessible WC to Part M requirements

- Upgrade/renew plumbing, drainage & ventilation as needed with the objective of concealing/reducing visible pipework

- Electrical works incl. new circuits for dryers as required

- Provide tiles/washable wall finishes. Tiling to half wall of all bathrooms and full wall on men’s’ urinals. (Partition/cubicles will be considered as an alternative as noted below).

- Epoxy grouting for durability

- Final testing, commissioning, certification & handover.

6. SANITARYWARE SPECIFICATION

- 5 No WC pans: close-coupled or back-to-wall or concealed cistern, vitreous china (Illustration/manufacturer details to be provided)

- Accessible 1 No WC with compliant grab rails, back support, alarm, fittings.

- 2 No Urinals: wall-mounted with water management (manufacturer detail to be provided)

- Basins: wall-mounted with mixers or within units (Illustration/manufacturer details to be provided).

- Partition/cubicle sets will be considered as an alternative/addition. Sample and illustration to be provided.

- Hand dryers: high-efficiency

7. ELECTRICAL WORKS

- LED lighting and PIR control

- Accessible WC alarm

- Rewiring as required; to BS7671

8. MECHANICAL VENTILATION

- Install extraction to WCs; Building Regulations compliant.

9. FLOORING

- Commercial grade slip-resistant seamless flooring with coved skirting’s for mopping purposes to all bathrooms and foyer.

10.  DECORATION

- Decorate walls & ceilings with a washable paint.

11. ACCESSIBILITY

- Full Part M compliance for accessible WC.

12. SIGNAGE

- Any appropriate additional WC signage as may be required.

13. TESTING & COMMISSIONING

- Certify relevant plumbing, electrics, ventilation.

14.  HEALTH & SAFETY

- Comply with CDM 2015.

## 15. QUALITY & INSPECTION

## Progress reviews; snag resolution prior to handover.

## 16. REGULATIONS & STANDARDS

All work must comply with current UK Building Regulations, including but not limited to:

- Part M (Access)

- Part G (Sanitation)

- BS 6465-1 & BS8300

- WRAS-approved fittings

- Electrical works to BS7671

## 17. TIMESCALE

- Contractor to provide programme of works.

## 18. WARRANTY

- Minimum 12 months on workmanship + manufacturer warranties.