Residual Waste Mobile Material Shredder:  scope of requirements

Lancashire Renewables Ltd currently deploy a mobile shredder (Komptech Terminator 5000 S) at our Leyland facility; processing circa 60,000 tonnes per annum of incoming household “black bag” and municipal bulky waste, to produce a combustible grade of waste stream, as output.

This equipment is now reaching “end of life” and we intend to conduct a tender to procure a replacement package on a long-term contract plant rental scheme: The supplier will also be required to take full responsibility for a complete maintenance package, that ensures our continuity of production Mon – Sat, over 52 weeks p.a.

The shredder must be designed and built to withstand the rigours of a working life in the household and bulky waste processing industry, in an enclosed operating atmosphere.

Term of Contract Hire:

A three-year period.

Start Date:

To be determined; but this will be subject to the lead time and availability of suitable equipment.

(01/04/2022 is indicative)

Rates & Payment Terms:

A fixed monthly rate (contract value / 36 months) is required for the entire contract period; and is to be fully inclusive of any additions to the standard equipment specifications, along with the costs associated with a full scheduled maintenance plan.

Equipment Specification / Criteria:

- Equipment must be configured to process household “black bag” waste, and municipal bulky waste.

- Mobile platform, utilising wheels or tracks.

- Diesel or electric drive equipment may be considered.

- Equipment can be new or of significantly low run hours with full maintenance history.

- Slow speed / single shaft.

- Remote control functionality.

- Throughput capability – between 40 -  80 tonnes / hour.

- Output fraction size – sub 300mm

- Over band magnet on material discharge belt.

- Automatic fire suppression equipment installed.

- Automatic lubrication system is preferred.

Service & Maintenance:

The equipment is fundamental to our continued daily processing of waste and the supplier must take this into consideration when managing scheduled or unplanned maintenance.

- Our typical daily usage is six hours between 07:00 – 18:00 (Mon – Fri), 07:00 – 12:00pm (Sat)

- In the event of unplanned maintenance / breakdown, we require a maximum four-hour response time to attend to the equipment.

- In the event of an equipment failure, deemed to be the supplier’s responsibility, which cannot be resolved within twenty four hours of the supplier’s Engineer attending the breakdown; the supplier must provide equivalent substitution equipment for the remaining duration of the repair work.

- For convenience, on site storage can be arranged for contingency stocks of replacement parts & consumables.