
SSP/00148
Schedule 13 - Palletisation

CONTRACT REQUIREMENTS FOR RETAIL TRADE AND BULK RECEIPTS FROM INDUSTRY WHERE GOODS ARE TO BE PALLETISED FOR  PCE DT CONTRACTS

1.
Applicability

a.
This requirement is applicable to receipts of 1 pallet or more of clothing or footwear and other PCE Team stores, which due to their dimensions or characteristics, can be safely configured in a palletised format.

 b. All bulk and trade receipts which consists of one or more pallet load must be presented to DSDC on the 
     NATO standard one tonne pallet conforming to drawing AO/11125 28 Jan 98 attached. Wood used in 
     pallets must comply with the provisions of  paragraph k of Condition B9, Timber and Wood-Derived Products,    and be marked so as to conform with the provisions of the International Standard for Phytosanitary Measures, Guidelines for Regulating Wood Packaging Material in International Trade, Publication No 15.     
2.
Modular Pallet Load Packaging Requirements.

a.

Over packing onto the pallet must not exceed the base pallet dimensions of 1200 x 1000mm and the maximum permissible height of the load including the pallet must not exceed 1030mm. Maximum loaded weight including pallet must not exceed 500kg.

b.
The loaded pallet must have a flat top in order that a second and similar load can be placed on top to facilitate safe transportation and mechanical handling in transit without damage being incurred to either pallet or loads.

c.
When securing the load to the pallet the contractor must be aware that the goods will be transported and handled many times before reaching their final destination to the user.  The robustness of keeping the pallet load intact is of paramount importance.

3.
Segregation

a.
Each pallet load is to contain packages of the same Nato stock number and size as the product contract description and must be labelled as “FULL PALLET”.

b.
Where the final demand quantities exceed the full load requirement, several Nato stock numbers from a single demand and from the same contract can be palletised together, these must be clearly labelled “MIXED PALLET”.

c.
Where a delivery includes a pallet load that does not constitute a full pallet load, then that pallet must be labelled “PART PALLET”.

4.
Securing Packages To Pallets.

a.

More than one method will be allowed to secure packages to a single pallet load i.e. non-metallic strapping or stretch wrap or a combination of both. When using non-metallic strapping a minimum of four straps will be used in cross-fitted positions. Two or more pallet loads must not be fastened together.

5.
Documentation

a.
All pallets must be pallet wrapped using a clear wrapping, which catches a single pallet summary sheet giving full identification.

b.
The package/pallet load that contains the consignment documentation must be clearly marked DOCUMENTATION HERE.

	
	Page 1
	

	
	
	



