ProCure22 Framework Agreement

P22 Framework Instruction 7: P22 NEC3 Contract Templates - Building Information Modelling (BIM)

Employers Information Requirements (EIR) Explanatory Notes

Introduction.

The notes below relate directly to Appendix 5 in the P22 NEC 3 Option C Templates B/D/F Under the Government Construction Strategy use of BIM is mandatory on all public sector capital schemes.  The notes are provided for guidance in the application of the EIR but do not take precedent to or overrule the text of Appendix 5.

There is a Jargon Buster (set of Definitions) at the end of this document to assist with some of the new terms that incorporation of BIM can bring.

The P22 Works Information EIR is to provide the Client with comprehensive contractual BIM requirement that defines the BIM specification and process for a project. The EIR provides for the Client that does not have in house BIM expertise but requires the implementation of BIM in line with Government Requirements. It is designed to be a robust minimum that can be elevated in scope in agreement with the PSCP.  The EIR is designed to be cost neutral, deviation may have cost implications therefore implementation must be discussed and agreed with the PSCP prior to production of a BIM Execution Plan.

The benefits of BIM are detailed in the copy of the P21+ BIM Executive Summary  included at Appendix A.

Before considering the EIR the Client should consider the P22 BIM Benefits Matrix (see Appendix B) to determine their high level BIM requirement for the project.  The Benefits Matrix has been jointly developed with the PSCPs and represents a standardised view of BIM deliverables for P22 projects.  All PSCPs have all signed up to the content and implementation of the EIR to deliver the benefits stated, albeit the method with which they do so will be unique to them.

If you have any comments on the EIR please contact your P22 IA and/or DH. _________________________________________________________________________________

- Overview

The EIR requires editing to confirm project-specific requirements for each of the sections. Standard responses have been included and in many cases these can be adopted without amendment. However, all EIR content requires review and confirmation prior to issue.

If BIM is not to be used on the project the Client Project Manager will cross out Appendix 5.

The EIR assumes PSCP appointment at Stage 2 OBC, with the first BIM output (Data Exchange) being at stage 2.

The EIR is an evolving document to be updated as BIM Level 2 standards evolve.

Areas where project specific adjustments are likely to be required include:

- Type of asset. This document is prepared on the assumption that the asset is the construction and/or refurbishment of a capital asset on the Major/Small Works Contract Templates;

- Project stages. Data exchanges and associated information requirements are to be mapped against the P22 project stages. Where implemented in a Small Works Scheme guidance is to be sought from P22 Helpdesk for Data exchanges at the 2 key stages of each Task.

- Information requirements. Data exchanges are to be aligned against the needs of the project – e.g. new-build vs. on-going asset management;

- Procurement strategy. Data exchanges are to be aligned with the procurement strategy as noted;

- IT requirements. Collaboration tools and other employer-specific requirements are to be specified. Client IT tools referred to in the EIR are indicative and not a requirement to use specific products e.g. a collaboration site provided by the employer or PSCP;

- Terminology is to align with P22 terminology and refer to P22 Guidance / Contractual documents where required;

- Project specific adjustment may be required in response to specific data requirements to be handed to the Client or input into an existing CAFM system at the end of the project.  However a robust minimum requirement is included in the EIR;

- Nothing in the EIR  overrides the requirements of the P22 National Framework Agreement and its Appendices;

Note:  The Client is to ensure the above requirements are applied throughout the contract.

1.1 BIM Strategic Objectives

Before completing this section the Client should refer to the P22 Benefits Matrix the and replicate the selections within the EIR.

1.2 Project Specific Challenges

The Client summarises the BIM status at the point the PSCP is selected and any qualifications on its administration. For example Client restrictions and limitations - supplier input / liaison required such as Computer Aided Facilities Management (CAFM) requirements, limits on the geometric or Level of Detail / Level of information required.

2.1 Standards

This specifies the BIM Task Group standards to be applied to the scheme. Additional standards used by the PSCP are to be defined within the BEP.  The stated BS/PAS standards have been developed by the BIM Task Group and represent a robust standard for BIM Implementation across the public sector. The standards are subject to continual development by the BIM Task Group; such development is to be incorporated in the project specific EIR.

2.2 Roles and Responsibilities

The purpose of this section in the EIR is to bring to the attention of the project team the allocation of roles associated with the management of the BIM model and project information. It’s important to note that this is not necessarily additional information being produced for BIM specifically, just a change in format of the project information already produced normally.

The roles are envisaged as being undertaken by individuals as part of their normal duties within the Project team and are not seen as additional roles for additional staff.

Details of the scope of services of the Information Manager are as prepared by the Construction Industry Council. An outline version suitable for incorporation with any appointments is published on the BIM Task Group website http://www.bimtaskgroup.org/wp-content/uploads/2013/02/Outline-Scope-of-Services-for-the-Role-of-Information-Managment.pdf

PAS 1192-2 section 7.5 table 2 provides a useful cross-tabulated summary of the roles as they apply across Project Team Members.

The PSCP is to consider these standards and adopt them for the project in context with the EIR for inclusion in the BIM Execution Plan.

2.3 Planning the Work and Data Segregation

This section sets out requirements for the management of the modelling process.  The PSCP will respond in their BEP.

Where the employer has specific requirements for work management, this should be identified and discussed with the PSCP prior to submission of the BEP.

The PSCP BEP includes recommendations on:

- Model Management:

  - Details for procedures required.  The expectation is that this will be co-ordinated by the Information Manager (PSCP).

- Volumes, Zones and Areas:

  - Requirements in connection with the definitions of zones and the management of adjacency within the model.

  - Requirements in connection with the definition of the project volume structure.

- Naming Conventions:

  - Definition of the requirement for a single project convention.

2.4 Security

Record Client specific security measures required in order to secure the data and model.

2.5. Coordination and Clash Detection

This section defines the required co-ordination process, together with requirements for quality control. This should be aligned with design development meetings where the latest version of the BIM model is used.

2.6 Collaboration Process

Defines how the Client, PSCP and Supply-Chain will collaboratively develop the model.  The  requirements will be reflected in the PSCP’s BEP.  The Client needs to ensure that they have the latest version of the model available to them in periodic design development meetings and at key decision points.  If possible amendments should be made directly  to the live version of the model in a Common Data Environment.

2.7 Health and Safety / Construction Design Management

The PSCP states how BIM based working will support H&S/CDM monitoring.

Clients should consider their requirement for H&S/CDM information to be provided in the BIM model, during design and construction stages. The Client should require access to this information in whatever format it feels best to receive it rather than requiring it to be part of the BIM model specifically.

Consideration should be given as to how post-construction Health and Safety information is to be recorded where Client does or does not require the BIM model to be used post construction for FM.

2.8 Systems Performance

Requirement is for the PSCP to ascertain whether or not the Client IT system has BIM capability. If the answer is no then PSCP proposes suitable measures.

2.9 Compliance Plan

The PSCP records and communicates how the integrity of the model and other data sources are to be maintained.

2.10 Delivery Strategy for Asset Information

This section defines the information exchange standard for Asset Information delivery into the employer’s FM system for ongoing FM Management at the end of the project.

Clients should consider carefully the information they require and how they may use it.  The P22 Standard Core Data-set provides a basic level of information that offers the following information for each component.  If the Client requires more information, they should discuss this with the PSCP in development of the EIR.

- Name

- Description

- Reference Number

- Use Classification

- Material and Finish Information

- Manufacturer

- Serial Number

- Technical information for Mechanical or Electrical Equipment

- Location

- FM Asset Number

- O+M Reference

- Residual Risks /Health and Safety Information

The requirements could be extended to identify specific requirements for cleaning, renewable assets, life expiry dates, statutory compliance certificates etc, if required. However Clients need to be mindful that asking for more information can lead to a very large data set and file size that can be difficult to operate.  Expanding the range of information requirements may increase the cost of delivery.

The Client should identify their CAFM system in Section 1.2.

- Technical Requirements

This section establishes technical information requirements, including the available Client software, data exchange contents and required level of detail.

The PSCP consults with the Project Manager identifying additional hardware/software the Client may need to implement BIM for the project

  - Software Platforms

It is necessary to ascertain at an early date in the application of BIM which software the Client has available and whether it is compatible with that operated by the PSCP. Final decisions on compatibility are to be recorded for avoidance of doubt.

  - Data Exchange Format

This section defines the formats used to deliver data at data exchanges.

Government (BIM Task Group) specifies the requirement for information in a COBie format. This is the most commonly used and understood format.  The Construction-Operations Building Information Exchange (COBie) format facilitates the delivery of Asset Information during planning, design, construction, and commissioning for delivery to facility owners and operators.  It is commonly delivered in an MS Excel spreadsheet.

  - Co-ordinates

A common co-ordinate system is stipulated for all BIM data with consistent adoption for all models. It ensures that all building components are located relative to an agreed point of origin. The PSCP uses a common co-ordinate system for all BIM data which is to be described within the BEP. If project-specific co-ordinate requirements exist, these should be inserted.

  - Level of Definition

Defines the requirements for information submissions/data exchanges at project stages. This information is used to develop the Master Information Delivery Plan (MIDP) and also defines, for the purposes of the contract, the levels of detail used for the various phases of the project.  As the project develops, increasing levels of detail can be included in the model.

3.5. Training

The PSCP provides training in application of the software and process required for the operation of BIM for Client staff.

The PSCP is required to provide trained personnel to operate and deliver the BIM process.

4. Data Exchanges and Project Deliverables

Defines the content of data exchanges and how the exchanges are aligned to work stages.

P22 PSCPs have agreed a standard minimum data set that to be supplied with all BIM projects,

To plan the development of the model and the design, the PSCP provides a Model Production Delivery Table (MPDT) in their BEP that specifies who will produce the information, when and in what detail.

- Jargon Buster

Data Exchanges – sets of data to be uploaded to the model at key decision points in the process.  Under P22 these are at SOC, OBC, FBC, and Completion.  See Section 6 in the EIR for the P22 standard Client requirements for data exchanges. Data Exchanges were previously known as ‘Data Drops’.

BEP – BIM Execution Plan.  The PSCP develops the BIM Execution Plan to describe in detail how they execute BIM on the scheme. Client engages with the PSCP in its development and acceptance.

MPDT – Model Production Delivery Table (MPDT).  The PSCP supplies this in their BEP and sets out all the models that will be developed and at high level who is responsible for the supply of discipline based information, and when.

MIDP – Master Information Delivery Plan.  The PSCP develops this plan with their suppliers to organise the delivery of asset data to the model. It specifies all the information deliverables on the project including but not limited to drawings, specifications, equipment and room data sheets. It records who, what, how and when information is to be supplied by linking it to the design and the programme.

COBie – Construction Operations Building Information Exchange.  This is the data format recommended by Government for the recording and communicating asset data.  It is usually in MS Excel format and supports the Federated BIM Model.  It can be exported from the BIM model and used in a CAFM system.

IFC – Industry Foundation Classes. An open file format that facilitates the interoperability of different BIM software for different disciplines.  BIM software can export models in IFC format that can then be imported by different software to federate the models.  The Government are supporting the use of IFC as standard, however there are issues with compatibility in some cases which the PSCP is to resolve for the project.

Federated Model – A BIM Model is collection of several different discipline based models that are Federated together to create a single interface for the Client.  When the models are federated the software checks for clashes and other inconsistencies and provides a report.

APPENDIX A

P21+ BIM Executive Summary

APPENDIX B

ProCure22 BIM Benefits Matrix

| POTENTIAL BENEFITS | Weight /10 | BIM Model features to enable benefits | Pre-requisites and other notes | Client Role to enable benefits | Yes/No |  |
| --- | --- | --- | --- | --- | --- | --- |
| See potential benefits below of using BIM | Score importance of the benefits for your scheme | See features of a BIM model that need to be present to enable the benefits to be delivered. |  | To get the most benefit from BIM, the Client must engage in the design development process considering BIM. The notes below show the key client roles and responsibilities. | Indicate whether you want this benefit on your scheme |  |
| - Visualisation |  |  |  |  |  |  |
| - User Engagement – Clear, easier understanding by all stakeholders incl. Clinical staff, Service managers, Patients, FM etc. of the design and potential design options. - Incomplete briefing /design short comings identified in advance of construction and addressed. - Transparency – ownership by all parties - Enablement of better and faster design decisions. - Reduced time for clinical staff away from front-line services. - Cost Neutral – 3D visualisations are a free by product of using BIM software. |  | - All visuals are generated from the working 3D BIM Model. - The 3D model generates all drawings, area schedules, Room Data Sheets (RDS), with specifications, finishes and performance requirements linked to the model etc. - Fully loaded Room layout drawings align with 3D model, with coordinated architecture, services and structure and using manufacturer objects. | - 3D BIM models are produced to BIM standards – BS1192:2007 and PAS1192-2:2013, BS8541 for library objects etc. - Visuals align with 3D BIM model and 2D drawings, schedules etc. - Visualisations can be produced ‘on the fly’ to inform design meetings. - Visuals are produced from the BIM model directly, showing clashes etc. CGI models are often enhanced or based on out of date information. - Approval of fully coordinated layouts will result in fewer changes at later stages which are much more costly. - Changes to the 3D BIM model automatically update 2D drawings. - Quantities for costing can be carried out from the model more easily for optioneering - There should be no disclaimers from the consultant team that the model is for information only. Notes: Sketch-up and 3D Studio visualisations are not BIM models as they do not have data to support them. | - Ensure that the Employers Requirements (ERs)are included in the Works Information (WI) with defined with documented processes for compliance. This should be done with the PSCP. - Ensure that the project drivers are well documented and understood by the whole team. - Ensure that the ERs (WI) are accessible by the whole team and managed via a Common Data Environment (web-based system) - Ensure that the team work collaboratively for coordinated designs with sharing of information via a Common Data Environment. - Ensure that the Client team have sufficient IT resource/permissions to view BIM models and/or have stated file types that can be viewed. Clients can view models in the following: - ‘free viewers’ eg; Navisworks Freedom, BIMX, Solbri, Tekla etc. - Paid for subscription viewers such as BIM360 which will work on iPads etc. - Ensure that the Design team Scope of Services includes: - Coordinated roles and responsibilities in line with the requirements of this benefits matrix. - requirement for 3D BIM models in line with the ERs - 2D drawings are produced and align with the BIM model - Model coordination – this may be the Lead Designer or BIM Manager. - Rendering of CGi visuals from the model - Require that design workshops use a live model and integrate all aspects of the project requirements |  |  |
| - Clinical planning |  |  |  |  |  |  |
| - Use 3D models to inform clinical planning/patient pathways/departmental adjacencies. Models provide much better levels in formation for clinical stakeholders, leading to more efficient use of their time, quicker decision making and optioneering. - Less clinical down-time. - Reduced stakeholder engagement time allowance in project. - Optimum Patient journeys can be clearly identified in advance, and information captured to confirm/inform or help derive departmental operational policies. - Potential Staff travel distances captured and minimised. - Improved information/understanding of sightlines. |  | - Alignment of 3D model with drawings, area schedules, RDS, specification etc. - Use of BIM modelling software plugins to manage adjacency planning, travel distances, RDS etc; eg Simulex, Affinity. | - The area schedules must be generated from the BIM model. - Simulations align with 3D BIM model and 2D drawings, schedules etc. - Visuals are produced from the BIM model directly, showing clashes etc. CGi models are often enhanced or based on out of date information. | - Ensure that the Design team Scope of Services includes: - Adjacency planning and RDS management - Travel analysis - Get clinical buy-in to BIM 3D visualisation |  |  |
| 3. Clash Detection |  |  |  |  |  |  |
| - Increased design integrity, giving client and PSCP assurance of the total design solution, construction solution and quality of the final product. - Reduced risk in the project for lower GMP. - Reduction in delays from services/structural clashes etc. unforeseen and reduced costly site fixes. - Reduced re-design. - Reduced programme. |  | - All design disciplines use BIM software from outset – eg Structural, Architectural and M+E models. - File formats are agreed at outset in BIM Execution Plans, including Federation software. - Alignment of 3D model with drawings, area schedules, RDS, finishes. | - 3D BIM models are produced to BIM standards in accordance with BIM Task Group guidance. - Models are exchanged and coordinated through use of a Common Data Environment in line with eg 4Projects, BIM, Buzzsaw etc. - Information is managed, developed and exchanged in line with the Project programme, including use of Information Release Schedules. - Information should be checked as suitable for issue in line with requirements and latest accepted design to avoid abortive work. | - Ensure that the Design team Scope of works includes: - Use of BIM for all disciplines - Coordinated roles and responsibilities in line with ERs - 3D coordination and/or clash detection either by the Lead Designer and/or BIM Consultant - Design deliverables for each project stage - Ensure that the procurement strategy included in the P22 Contract Template specifies BIM information requirements to be included in work package tender information.. - Early engagement of contractor and specialist subcontractors. - Ensure that the team work collaboratively for coordinated designs with sharing of information via a Common Data Environment. |  |  |
| 4. Standardisation |  |  |  |  |  |  |
| - Use of ProjectShare and StandardShare for quicker model production - avoids re-design fees. - Benefit from cost reductions of Repeatable Designs and Standard Components. - Reduction of clinical engagement burden on clinical staff and associated efficiency benefits. - Spend to save options from earlier projects can be readily tested/incorporated. |  | - Use of BIM Library objects to BIM standards – finishes, FF&E, MEP etc. - Library objects contain all relevant data - Shared web accessible library. - Room Layouts established and set up as Families/Groups eg ADB, Codebook, Roombook etc. | - Library objects built to BIM Standards. - P22 Repeatable Designs and Standard Components are BIM compatible. - Manufacturer engagement to produce object families. | - Ensure that P22 Repeatable Designs and Standard Components are specified for use in the P22 Contract Template. - Ensure that the ERs include requirement to include standard BIM compatible products, systems and layouts. - Require that the consultant team have access to standardised libraries. - Require that the design team engage with manufacturers to develop BIM library objects as required. |  |  |
| 5. WLC Costing /Asset Data for FM (6D) |  |  |  |  |  |  |
| - More efficient maintenance of the building throughout its life. - The client can retain a full digital model of the finished project, compliant with their CAFM system. - Improved knowledge of lifecycle issues and costs in the design period for optimum specification taking WLC issues into account. - Permanent record of all components and specifications, warranties (or those specified) in the building with maintenance/replacement schedules, with life cycle costs. - Early engagement of the FM team in the design stage for improved design, engagement and communication on Lifecycle issues that in the long-term will be far in excess of construction cost. - Optimise and re-model impact of component/design lifecycle revenue costs including Energy. - Improved operational efficiency through access to accurate readily accessible digital information. - PPM Lists. |  | - Alignment of 3D model with 2D drawings, area schedules, RDS, specification etc. - All design disciplines use BIM software from outset. - Component BIM data included in the model according to client requirement. - Specified levels of detail for components for consistent information and tendering - Environmental performance data. - Inclusion of Equipment schedules, warranties, maintenance schedules, component tagging, user guides - Model integration with CAFM system (if required). | - 3D BIM models are produced to BIM standards, including Library objects, with component data - Use of BIM data to establish principles of the O+M requirements and lifecycle analysis | - Early engagement of the FM team - Use of lessons learnt from existing estate, including products, access, ease of maintenance etc. - Specification of the most common FM issues to support where additional detail will be required in the model. - Ensure that the ERs include reference to model requirements as appropriate – such as; - Room occupancy density and time periods, specialist equipment etc for energy loading, - Environmental Comfort standards and targets, - Lifecycle Analysis, - EPC requirements and/or energy use targets, - Expected BREEAM performance - Zero Carbon, - Consideration of product FM and life expectancy, - Information required for O+Ms – Equipment schedules, warranties, maintenance requirements for PPM, RFID barcode tagging etc, - Requirements for interfacing with a CAFM system, - Confirm if existing CAFM is BIM ready, including model integration and COBie data, - Requirements for Soft landings, Building User Guides, Training, aftercare, - Confirm requirements for hand-over data format– file types, AS-built surveys etc. |  |  |
| 6. Procurement |  |  |  |  |  |  |
| - Improved accuracy for fast consistent work package tendering. - Better understanding of design and client requirements for the supply-chain for Improved certainty of cost/cost reduction opportunities. - Consistency of requirement across all work packages. - Reduction of risk. - Reduced wastage. - Improves information for suppliers to propose innovative options. - Provision of additional component information that may not be captured otherwise. |  | - Alignment of 3D model with 2D drawings, area schedules, RDS, finishes. - All design disciplines use BIM software from outset. - Clash detection has been carried out - All models are accurate and contain relevant data. - Use of standardised and rationalised components/library objects etc including product interfaces, dimensional compatibility. | - Models and tenders can be exchanged and coordinated through use of a Common Data Environment. - Use of specification tools & Plugins – NBS Create - Information is managed, developed and exchanged in line with the programme and P22 stages. - Suppliers must be BIM capable. Additional charges for BIM component information should not be considered appropriate. Accepting that if a supplier with a superior product but no BIM capability is preferable to a client, then the supplier must be encouraged to invest in BIM. | - BIM models are issued with tender information - Employer’s Information Requirements are fully defined to allow for validation of the ER/Brief. - Sub-contracts reference BIM deliverables as set out in ERs. There should be no need to amend the P22 NEC3 Template - Cost Manager must able to use BIM models for quantity take off for cost planning and/or BoQ. - Design team Scope of Services should include: - BIM employer requirements, - Fully defined BIM roles and responsibilities, - Requirement for data rich models, - Models produced to BIM standards, - Models will be used for Tender. - Early engagement of specialist contractors and supply chain. - Identify KPis to be monitored/achieved for PSCP performance management of the supply-chain. |  |  |
| 7. Future Change of use |  |  |  |  |  |  |
| - Supports refurbishment projects - Allows cheaper and faster information to support optioneering for future proofing. - Early identification of design clashes particularly for M+E - Site Surveys/cloud scanning can survey existing site for refurbishment - Flexibility |  | - Alignment of 3D model with 2D drawings, area schedules, RDS, finishes. - All design disciplines use BIM software from outset. - Clash detection has been carried out on the base design. - Design appraisal workshops to develop optioneering that considers all disciplines. - Additional Clash detection carried out on options. - Data rich models for cost analysis of options. | - 3D point cloud surveys carried out of existing buildings, converted to 3D BIM models, checked and validated for accuracy. | - Ensure that the ERs include reference to: - Flexibility and future proofing, - Interfaces with existing buildings, - Requirements for surveys. |  |  |
| 8. 4D and 5D (incorporation of Cost and Programme) |  |  |  |  |  |  |
| - Provides of granularity of programme and cost for the Trust – quick access on shared model. - Allows real-time update. - Better site/supplies coordination for Improved programme. - Whole team visibility / understanding of programme. | 4D 5D | - Alignment of 3D model with 2D drawings, area schedules, RDS, finishes. - All design disciplines use BIM software from outset. - Fully resourced programme, linked with cash-flow, P22 stages and key dates. - Clash detection has been carried out on the base design. - Data rich models to identify materials and quantities for 4D and 5D simulations. | - Existing infrastructure also modelled . - Fully resourced programme. - PSCP/Supply-Chain and client advisor ability to use BIM plug-in software. | - BIM models are issued with tender information. - Employer’s Information Requirements are fully defined to allow for validation of the ER/Brief. - Require monthly updates of cost and programme supported by a live BIM model. |  |  |
| 9. Supply Chain |  |  |  |  |  |  |
| - Improves design co-ordination and collaboration in a real time shared environment – for reduced design programme (each discipline can work on the design simultaneously over the web). - Reduced design meetings and associated travel time and costs. - Improves opportunity for innovation. - Gives incentive for price reduction. |  | - Alignment of 3D model with 2D drawings, area schedules, RDS, finishes. - Clash detection has been carried out on the base design. - Use of standardised and rationalised components/library objects etc. | - The supply chain must understand and accept the benefits of BIM for their own business. - Supply Chain BIM capability is assessed – PAS91 etc. - Specify BIM data as a requirement in Procurement Strategy. - All design disciplines use BIM software from outset. - BIM awareness and training may be required by supply-chain. | - BIM models are issued with tender information. - Employer’s Information Requirements are fully defined to allow for validation of the ER/Brief. - Early engagement of specialist contractors and supply chain. - Contractor Design elements fully defined with roles and responsibilities. - BIM deliverables included in Tender requirements. |  |  |
| 10. Energy Analysis |  |  |  |  |  |
| - Better information to support options appraisal and investment opportunities. - Assurance of energy performance and associated costs. - A reduction in Carbon Emissions and lower energy costs. |  | - Optimise and re-model impact of component/design lifecycle revenue costs including Energy. - Use of the BIM model for energy, thermal, daylighting, carbon etc analysis. | - Models must have all spaces identified for energy analysis. - Elements must be modelled correctly for effective and accurate analysis, including correct u-values for the external envelope. | - Confirm BREAM performance requirements. - Details particular design features/equipment to be included. - Ensure M+E designer has BIM Energy Modelling capability. |  |