FAO Ms Holly Mann

Dear Holly

Thank you for allowing me to visit your play area on Monday to measure the muddy play areas.

As discussed I have quoted for our strongest most durable play grass Stadia 24. This is designed to take adult competitive play so it very suitable for this application. There are cheaper alternatives.

As discussed, I have measured each area to around 100mm past the mud to ensure adequate coverage. This has produced my total square meterage. Please see the table below for reference.

For all areas except the bark area under the climbing frame (area 5) and under the rope tree equipment (area 10). Our price includes:

- Removal and safe disposal of 150mm existing mud/turf

- Installation of 100mm compacted MOT stone

- Construction of a concrete haunch to all perimeters to stick down to. (we do not use pins in play areas)

- Installation and compaction of a 35mm sharp sand base.

- Installation of Geotextile weed membrane.

- Installation of Nomow Stadia 24 artificial grass including trimming to perimeters and any stumps or posts.

- Removal of all waste, leaving the site clean and tidy.

Total 12 Play Areas – 141 square metres - £10,884.00

For all areas under the climbing frame (area 5) and under the rope tree equipment (area 10). Our price includes:

- Removal and safe disposal of 150mm existing bark mulch, mud/turf

- Installation grading & compaction of 100mm MOT stone

- Construction of a concrete haunch to all perimeters to stick down to. (we do not use pins in play areas)

- Installation of 45mm EcoSafe shock pad to area 5 to provide certified 2.00m critical fall height 111m2

- Installation of 35mm EcoSafe shock pad to area 10 to provide certified 1.5m critical fall height 34m2

- Installation of Geotextile weed membrane.

- Installation of Nomow Stadia 24 artificial grass including trimming to perimeters and any stumps or posts.

- Removal of all waste, leaving the site clean and tidy.

Areas 5 & 10 – 145m2 - £12,957.00

I have attached a specification sheet and some indicative images.

All prices are plus VAT & fixed for 3 months.

All our installers are DBS checked.

I hope that this quotation is of interest. Please do come back to me if you need further information assistance.

All our artificial grass surfaces are designed and manufactured in Belgium to the highest European Standards and Specifications which is why we can offer a 10 year warranty.

A quotation pack containing small representative samples will follow by post. Please do not hesitate to contact me if I can be of further assistance.

|  | Equipment | Length M | Width M | Shape | M2 | Perimeter M |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Slide | 5 | 3 | Rectangle | 15 | 16 |
| 2 | S/S Spinner | 2.5 | 2.5 | Circle | 4.90875 | 7.854 |
| 3 | Swing Set | 3.5 | 4 | Rectangle | 14 | 15 |
| 4 | Dome swing | 4 | 5.5 | Rectangle | 22 | 19 |
| 5 | Large Climbing Frame | 10.3 | 10.8 | Rectangle | 111.24 | 42.2 |
| 6 | Trim Trail | 8.8 | 1 | ZigZag | 8.8 | 19.6 |
| 7 | Large Spinner | 3.5 | 3.5 | Circle | 9.62115 | 10.9956 |
| 8 | Hoop Swing | 3 | 3.4 | Rectangle | 10.2 | 12.8 |
| 9 | Stand Up Spinner | 3.5 | 3.5 | Circle | 9.62115 | 10.9956 |
| 10 | Rope Tree | 5.8 | 5.8 | Rectangle | 33.64 | 23.2 |
| 11 | Tyre Swing | 5 | 5 | Circle | 19.635 | 15.708 |
| 12 | Toddler Swing | 3.5 | 5 | Rectangle | 17.5 | 17 |
| 13 | Springer Car | 2.5 | 2.5 | Circle | 4.90875 | 7.854 |
| 14 | Springer | 2.5 | 2.5 | Circle | 4.90875 | 7.854 |
|  |  |  |  |  | 285.9836 | 226.0612 |